FARI

Example Investigation Report

USB APID 0x06 Undersized Telecommand

Release source: 03-usb-apid-underflow-report.md

FARI v1.3.0  |  2026-08-23

This is the versioned release edition of the FARI documentation bundle. Canonical working sources for this release are stored under the same version folder.

INCONCLUSIVE Qualitative executive conclusion from the example report

## Document Control

Assessment ID: ASM-FARI-2026-003

Investigation ID: INV-USB-002

Report version: 0.3

Report maturity: Provisional

Report date: 2026-06-08

Source material: cases/03_USB.md (../cases/03_USB.md) and cases/usb_tc_apid_underflow_cmd.png (../cases/usb_tc_apid_underflow_cmd.png)

Evidence producer: Auditor; identity not supplied

Report author: FARI Report Author

Mission owner / risk authority: Not supplied

Framework profile: No external framework profile was required for this report.

## 1. FARI Result Card

| Field | Result |
| --- | --- |
| Conclusion | Inconclusive |
| Confidence | Low |
| Required Action | Extend Investigation, then Retest |
| Priority | Immediate before treating the USB command path as resilient to malformed packets |
| Scope Boundary | One malformed APID 0x06 telecommand constructed and sent through the supplied USB lab procedure; handler execution, memory corruption, persistent command loss, RF reachability, and on-orbit behavior excluded |

Rationale: The supplied screenshot demonstrates construction and transmission of an undersized APID 0x06 packet and no received bytes before timeout. The source excerpt supports a credible unsigned-underflow hypothesis. The supplied material does not demonstrate that the target handler executed, that an unsafe copy occurred, or that the target stopped accepting subsequent commands.

## 2. Executive Decision Summary

The source material identifies a credible unsafe-length calculation in a BROADCAST_MSG handler and demonstrates that a one-byte APID 0x06 packet can be sent through the USB lab procedure. This is enough to require an immediate, instrumented retest and defensive length validation.

It is not enough to conclude that denial of service, memory corruption, or code execution occurred. A timeout has multiple possible explanations, including no defined response, framing rejection, handler rejection, transport behavior, or target failure.

Decision required: Do not claim resilience or confirmed exploitation. Instrument the handler, prove packet acceptance and post-test command behavior, then resolve the claim.

## 3. Frame

### Mission Context Added by the Report Author

The supplied material does not identify the mission architecture. This report uses the explicit assumption that safe telecommand parsing and continued command handling support spacecraft commandability and recovery. Mission-owner confirmation is required.

### Scope

| Item | FARI Record |
| --- | --- |
| Primary asset | AST-USB-TC-002 USB-exposed telecommand parser |
| Related asset | AST-FSW-BCAST-001 APID 0x06 broadcast-message handler |
| Segment | Space-segment representative lab interface and simulated link |
| Access basis | Private, as stated by evidence producer |
| Environment | Static/offline source excerpt and dynamic USB lab execution |
| Claim | CLM-USB-002: Undersized broadcast telecommands are rejected without unsafe memory access or loss of command handling |
| Explicit exclusions | Actual RF delivery, on-orbit behavior, authentication path, exploitability beyond the supplied packet, and mission-level consequence |

### Authorization

No authorization reference, stop condition, test date, target revision, or evidence-handling record was supplied.

## 4. Acquire: Source-Material Intake Review

### Evidence Index

| Evidence ID | Source | Directly Supported Statement |
| --- | --- | --- |
| EVD-USB-003-001 | Supplied source excerpt | The documented handler calculation derives payload_total, subtracts two, and passes the result to memcpy |
| EVD-USB-003-002 | usb_tc_apid_underflow_cmd.png | A decoded TC packet for APID 0x006 with length field 0, data-field size 1, and data 00 was sent using the USB command |
| EVD-USB-003-003 | usb_tc_apid_underflow_cmd.png | The tool reported no bytes received before timeout |

### Evidence-Backed Facts

The supplied source excerpt contains the sequence payload_total = header.length + 1, msg_len = payload_total - 2, and memcpy(buffer_msg, data + 2, msg_len).

The screenshot shows the command targeting /dev/cu.usbmodemfsat3 with broadcast --payload-hex 00.

The decoded packet is a TC for APID 0x006 (BROADCAST_MSG), length field 0, data-field size 1, captured bytes 7, and data 00.

The screenshot reports that no bytes were received before timeout.

### Evidence-Producer Assertions

The arithmetic uses an unsigned length and underflows when the declared payload is shorter than two bytes.

The resulting copy can cause memory corruption.

The current lab build may stop accepting commands until reset or USB re-enumeration.

The weakness could become denial of service or code execution over RF.

### Report-Author Inferences

If msg_len is unsigned and the supplied code executes for the demonstrated packet, subtracting two from a value of one would produce a very large copy length.

The screenshot supports delivery by the sending tool, but not target-side handler acceptance.

The observed timeout is compatible with target failure, rejection, absent response behavior, or transport/tool behavior and therefore cannot establish denial of service by itself.

### Contradictions and Quality Issues

| ID | Issue | Reporting Decision |
| --- | --- | --- |
| GAP-USB-013 | The supplied code excerpt for the sender uses a TCP connection, while the reproduction screenshot uses a serial-device option | Require the exact executed tool revision and transport path |
| GAP-USB-014 | The text asserts persistent command loss, but no baseline, post-test command, reset, or recovery evidence is supplied | Treat denial of service as unconfirmed |
| GAP-USB-015 | The finding states that the library has DoS in APID 0x06, which is broader than the evidence | Exclude the generic conclusion and preserve the investigation as Inconclusive |
| GAP-USB-016 | The source excerpt has no file path, revision, type definition, or build context | Treat root-cause and exploitability analysis as provisional |

### Sufficiency Decision

| Question | Decision | Rationale |
| --- | --- | --- |
| Technical condition | Partial | Credible unsafe arithmetic is shown, but execution and memory effect are not demonstrated |
| External / operational reachability | Partial | USB-side transmission is evidenced; target acceptance, RF delivery, and authentication are unknown |
| Mission consequence | Insufficient | No loss of command handling, recovery behavior, or mission effect is evidenced |

## 5. Relate: Candidate Finding and Mission Context

### FND-USB-002: Undersized Broadcast Message May Produce Unsafe Copy Length

State: Candidate

Condition: The supplied handler excerpt derives a message length by subtracting two from a payload total without an evidenced minimum-length check.

Observed effect: A one-byte APID 0x06 telecommand was sent; no response bytes were received before timeout.

Unobserved but credible effect: Unsafe memory access, handler failure, or command-path interruption if the packet reaches and executes the shown code.

Preconditions: The demonstrated packet reaches the handler; the supplied arithmetic and types match the tested build; no upstream validation rejects the packet.

Technical confidence: Medium for the code-pattern concern; low for target-side execution and effect.

### Framework Mappings

| Mapping | State | Rationale |
| --- | --- | --- |
| CWE-191 Integer Underflow | Candidate | The source excerpt suggests subtraction below the minimum valid payload size; types and execution are not confirmed |

### Qualitative Mission Context

Given access to a command-delivery path and an undersized APID 0x06 telecommand, the handler may derive an unsafe copy length and interrupt command handling, potentially harming commandability or recovery.

This scenario is credible but not demonstrated. FARI therefore records an actionable Inconclusive result instead of a vulnerability score or a confirmed mission-level denial of service.

## 6. Inform

### Required Instrumented Retest

- Record the exact sender version, target revision, transport path, and packet bytes.

- Capture a successful baseline command and response before the malformed packet.

- Instrument handler entry, parsed lengths, copy length, rejection path, process state, watchdog activity, and memory-safety diagnostics.

- Send the malformed packet once under defined stop conditions.

- Send a known-good command immediately after the test and record the result.

- Record recovery, reset, and USB re-enumeration behavior.

- Repeat after adding a minimum payload-length check.

### Required Remediation

Reject payloads shorter than the handler's minimum required fields before subtraction or pointer arithmetic.

Use checked arithmetic and size-bounded copies.

Add regression tests for zero-, one-, and boundary-length payloads.

Record malformed-command counters and explicit rejection telemetry.

### Acceptance Criteria

All undersized APID 0x06 packets are rejected before copy operations.

No sanitizer, fault, reset, watchdog, or abnormal process event occurs.

A known-good command succeeds immediately after each malformed input.

The patched behavior is demonstrated in the representative command path.

### Evidence Gaps and Follow-Up Requests

| Gap ID | Requested Evidence | Why It Matters |
| --- | --- | --- |
| GAP-USB-017 | Handler source file, types, revision, and build flags | Confirm the arithmetic and applicability |
| GAP-USB-018 | Exact executed sender and USB framing implementation | Resolve the transport contradiction |
| GAP-USB-019 | Target logs, debugger/sanitizer output, and post-test command | Resolve handler execution and denial-of-service claim |
| GAP-USB-020 | RF-path and command-authentication evidence | Evaluate operational reachability |
| GAP-USB-021 | Mission recovery requirements and unacceptable outcomes | Evaluate mission consequence |

### Assurance Statement

CLM-USB-002 is Inconclusive with low confidence. The supplied material demonstrates malformed-packet construction and USB-side transmission and supports a credible unsafe-length hypothesis. It does not demonstrate target-side memory corruption, persistent command loss, RF reachability, or mission consequence.

## 7. Coverage

| Area | Coverage | Result |
| --- | --- | --- |
| Malformed packet construction | Tested and evidenced | APID 0x06, one-byte data field |
| USB-side transmission | Tested and evidenced | Sender reported packet sent |
| Target handler entry and unsafe copy | Not evidenced | Requires instrumentation |
| Post-test command handling and recovery | Not assessed | Requires baseline and recovery commands |
| RF delivery and authentication | Not assessed | No conclusion |
| On-orbit behavior | Out of scope | No conclusion |

## 8. Closing FARI Result Card

| Field | Result |
| --- | --- |
| Conclusion | Inconclusive |
| Confidence | Low |
| Required Action | Extend Investigation, then Retest |
| Priority | Immediate before malformed-input resilience is claimed |
| Next Decision | Resolve the claim after instrumented execution proves rejection, unsafe effect, or bounded recovery |